# Home Network Architecture Brief

Generated: 2025-12-04 03:43:27

## 1. Network Overview

Your current network is designed with a strong backbone using eero Pro 6E as the primary router and a two-layer switch topology using TP-Link unmanaged PoE switches. This supports your high-density smart home with over 100 devices, including 72+ Philips Hue lights, HomePods, Apple TVs, Home Assistant (HA Green), and more.

## 2. Topology Details

[ONT]

│

└─(2.5G)─> [eero Pro 6E]

│

└─(1G)─> [LS105G PoE]

├─ HA Green

├─ Apple TV

├─ Your Mac/PC

└─(uplink)─> [LS108GS PoE]

├─ Hue Bridge

├─ M3 Hub

└─ eero 6+
- ONT (Wall-mounted internet source)
  → eero Pro 6E (WAN port via 2.5 Gbps Ethernet)
    → LS105G TP-Link 5-port PoE Switch (1 Gbps LAN)
      • Home Assistant Green
      • Apple TV
      • Your main computer
      → LS108GS TP-Link 8-port PoE Switch (via uplink from LS105G)
        • Aqara M3 Hub
        • eero 6+ (wired backhaul)
        • Philips Hue Bridge

## 3. Technical Strengths

- Solid eero mesh routing with wired backhaul to eero 6+.
- Switches are daisy-chained correctly with a single uplink (no loops).
- PoE is used efficiently for low-draw devices (Hue, Aqara).
- Core services (HA, n8n, Apple TV) are hardwired for reliability.
- Smart home automations are being orchestrated using n8n with SSH into HA Green for real-time automation updates.

## 4. Secure Remote Access (Cloudflare Tunnel)

Cloudflare Tunnel was previously used and can be restored to securely expose services like:
- Home Assistant → ha.itsjeff.org
- n8n → n8n.itsjeff.org
- SSH (optional) → ssh.itsjeff.org

All endpoints can be protected via Cloudflare Access with Google login, requiring no open ports and full HTTPS support.

## 5. Recommendations

- Replace LS105G with a VLAN-capable switch if planning network segmentation.
- Segment network in future with VLANs (IoT, Admin, Personal) for better discovery and control.
- Use mDNS reflector when VLANs are implemented to preserve HomeKit/Thread visibility.
- Consider adding a UPS to maintain automation reliability during power loss.
- Maintain HA automation scripts in `/config/automations/generated/` for clean YAML separation.
- Optionally add `ha core check` validation step before reloading automations via SSH.

## 6. Final Notes

This architecture reflects a highly intentional, scalable smart home network designed with Systems Division clarity—focusing on stability, segmentation, and long-term adaptability.